- Recording with a SD card

- The mixer can record and playback via a built in SD card recorder.

- The recording is done "Pre Fader" (but post pre amp) for all channels. This means that the complete audio input is recorded and is not affected by the fader settings.

- Select a SD card formatted for StudioLive recording (FAT32) with at least 10 GB free space (the card label name should be STUDIOLIVE). You can check this on a computer.

- Insert the card into the slot under "LIVE RECORDING".

- Create a New Session for Recording

- Press the Edit button.

- This will bring up the Live Recording form. The total memory size of the card (not its free space) and its label name (STUDIOLIVE) will be displayed.

- Press the New Session button. This will bring up the New Session form.

- Enter the date (in the form of 19 May 2018) into the Artist field. Leave the other fields blank.

- Press the Create button. The recording Capture form is now displayed.

- Select the Channels to be Recorded

- At this stage the Remaining time shown on the Capture form will show 0:00:00.

- Press the Select button under Arm Tracks. A Channel Select form will now be displayed and all the channel Select buttons on the mixer will commence flashing.

- Select the channels that you wish to record by pressing the channel Select button on the mixer or by pressing the channel number on the Channel Select form (note that not all channels can be selected this way).

- The Select buttons for the selected channels will stop flashing and glow steadily. The remaining buttons will continue to flash.

- When you have completed selecting channels press the Done button and the Channel Select form will close.

- The Remaining time shown on the Capture form will now show the actual recording time available for this card.

- Check that the Remaining time shown on the Capture form is greater than the service time (allow a good margin!).

- To Start Recording

- When ready to record press the Record button on the Transport Control. The recording will take a few seconds to start.

- The Rec. Lock button automatically turns on.

- To Stop Recording

- First recall the Capture form by pressing the Edit button.

- Press the Rec. Lock (which turned on when the recoding commenced)) to turn it off.

- Press the Stop button on the Transport Control.

- Wait until the hourglass closes. This will take a number of seconds.

- When the hour glass has gone press the Save and Close button on the Capture form.

- These last three steps are essential as no recording will be saved if they are not done.

- Backup Recording to the Audio Computer

- Remove the SC card from the mixer and plug into the audio computer.

- Copy the entire folder from the SD card to the "StudioLive Recordings" folder onto the audio computer.